How to use DLL by Excel

http:\\www.smarthomebus.com

- First，log in SMART official website download the SmartBus Library DLL.

  - Download → Softwares

  - Click Softwares

  - Find “SmartBus Library DLL”:

- After the download is complete installation

  - Open

  - Install

  - Next>

  - Please remember the installation path.  (Next>)

  - Next>

  - Please wait

  - Successful  Installation

  - Two files, “SBUS.dll” and “SBUS.tlb”.

“SBUS.dll”:  It is for .NET platform.“SBUS.tlb”:  It is for VB6.0 and Delphi7 platform . (it just like the “COM” as )

- New Excel documents.

  - New Excel

  - Open development environment

  - Add  button

  - Right click select properties, Change the button show name

- Use SBUS Library.

  - Add SBUS DLL

  - Find the SBUS.dll and SBUS.tlb

  - Select SBUS.tlb

  - OK

- Write code, use DLL.

  - Double click on the button to enter the code editor

  - Code Reference. Corresponding to double click the button, edit event

    - Dim com As New Dimmer

    - Private Sub CommandButton1_Click()

    - com.localIPAddress = "0.0.0.0"

    - com.InitSocket SBUS.PortType_PT_LAN

    - End Sub

    - Private Sub CommandButton2_Click()

    - com.Single_Light_Control 1, 75, 255, 100

    - End Sub

    - Private Sub CommandButton3_Click()

    - com.Single_Light_Control 1, 75, 255, 0

    - End Sub

- Test，Control SMARTBUS devices.

  - First init socket, then open light, close light.

For more details, please read "How to program an application simple.pdf"

2014.09.01

Chandler